F3 Block — Rock Physics & Synthetic Seismogram

Multi-Well Automated Python Framework

Wells: F02-1  |  F03-2  |  F03-4  |  F06-1  (All Real .wll Data)

| 5 Modules | 5 Phases | 9 Figures / Well | 44 Total Outputs |
| --- | --- | --- | --- |

# 1. Overview

This framework provides a fully automated, end-to-end rock physics and synthetic seismogram pipeline for the F3 Block, North Sea. It reads real OpendTect V6.6 binary .wll log files for all four wells — F02-1, F03-2, F03-4, and F06-1 — then runs a 5-phase workflow producing publication-quality figures and CSV exports for every well.

The pipeline is designed to be data-agnostic: any .wll directory is auto-scanned, wells are identified by regex, logs are mapped via a mnemonic table, and all phases run without manual intervention. Adding a new well requires only dropping its .wll files into the directory and registering the well name in REAL_WELLS.

## Key Capabilities

| Capability | Details |
| --- | --- |
| WLL Parsing | Reads OpendTect V6.6 binary .wll files; auto-discovers wells by regex; merges logs to 0.15 m depth grid |
| Log QC | Spike removal (physical bounds), linear interpolation, median filter smoothing |
| Elastic Params | AI, SI, Vp/Vs, Poisson's ratio, K_sat, G_sat from Vp + Vs + RHOB |
| Gassmann Sub. | Full 3-step brine to gas substitution with fluid and mineral property databases |
| Synthetic Seismic | Zero-offset + pre-stack 0-45 deg gathers; Ricker & Ormsby wavelets in TWT domain |
| AVO Analysis | Shuey 2-term intercept-gradient; difference sections; R0/G attribute traces |
| Multi-Well | Cross-well AI vs Vp/Vs crossplots; log overlay figures for all wells |

# 2. Project Structure

f3_rockphysics/

├── run_pipeline.py              ← Single entry point, CLI orchestrator

├── modules/

│   ├── wll_reader.py            ← OpendTect .wll parser + WellLoader class

│   ├── rock_physics.py          ← Elastic params + Gassmann substitution

│   ├── seismic_engine.py        ← Wavelets, depth→TWT, RC, angle gathers

│   └── plotting.py              ← All 9 figure types, light/white theme

├── wll_files/                   ← Drop .wll files here (auto-discovered)

└── outputs/                     ← Generated figures and CSVs

## Module Responsibilities

| Module | Responsibility |
| --- | --- |
| wll_reader.py | Parses binary OpendTect V6.6 .wll files; auto-discovers wells by regex (F\d{2}-\d); merges all logs onto a 0.15 m depth grid; QC-cleans spikes, interpolates gaps, applies median smoothing. |
| rock_physics.py | Phase 2: computes AI, SI, Vp/Vs, Poisson's ratio, K_sat, G_sat. Phase 3: 3-step Gassmann (back-strip to forward substitute to recompute velocities). Contains fluid and mineral property databases. |
| seismic_engine.py | SeismicEngine class: depth to TWT conversion using interval Vp; Ricker & Ormsby wavelet factory; zero-offset RC; Shuey 2-term AVO at every interface and angle; pre-stack gather matrix. |
| plotting.py | 9 figure functions with a clean light/white theme for publication-quality outputs. Accepts well_name + save_path. Supports wiggle-fill, variable-density (VA), and multi-well overlay modes. |

# 3. Installation & Dependencies

## Requirements

- Python 3.10+

- numpy, scipy, pandas, matplotlib (scientific stack)

## Install

pip install numpy scipy pandas matplotlib

## Clone / Setup

git clone https://github.com/Amalendu17/Rock-Physics-Synthetic-Seismogram.git

cd Rock-Physics-Synthetic-Seismogram

# Place .wll files in wll_files/

# 4. Quick Start

All phases are controlled through run_pipeline.py. The script auto-discovers .wll files, loads all three real wells, and produces all outputs in a single command.

## Run All Wells, All Phases

python run_pipeline.py \

--wll_dir ./wll_files \

--out_dir ./outputs

## Selective Execution

# Skip seismic phase (faster for rock physics only)

python run_pipeline.py --skip_phases 4

# Only multi-well overlays

python run_pipeline.py --only_phases 5

# Single well

python run_pipeline.py --wells F02-1

## Command-Line Arguments

| Argument | Default | Description |
| --- | --- | --- |
| --wll_dir | ./wll_files | Directory containing .wll files |
| --out_dir | ./outputs | Root output directory |
| --wells | F02-1 F03-2 F03-4 F06-1 | Space-separated list of wells to process |
| --skip_phases | (none) | Phase numbers to skip (1-5) |
| --only_phases | (none) | Run only these phases (overrides skip_phases) |
| --f_dom | 40.0 | Ricker wavelet dominant frequency (Hz) |
| --no_real | False | Skip loading real .wll files |

# 5. Workflow Phases

The pipeline is divided into 5 phases, each producing specific outputs. Phases can be run independently using --only_phases or excluded using --skip_phases.

| # | Phase | Actions | Outputs |
| --- | --- | --- | --- |
| 1 | Data Loading & QC | Parse .wll files; merge onto 0.15 m depth grid; spike removal (physical bounds); interpolation; median filter smoothing; lithology assignment from LITH or GR log | 01_log_overview.png |
| 2 | Elastic Parameters | Compute AI, SI, Vp/Vs, Poisson's ratio, K_sat, G_sat, M_sat from Vp, Vs, RHOB; lithology-grouped summary statistics | 02_elastic_params.png 03_crossplots.png |
| 3 | Gassmann Sub. | Back-strip brine to K_dry; forward Gassmann with gas (K_fl = 0.04 GPa); mass-balance density update; recompute Vp_gas, Vs_gas, AI_gas, Vp/Vs_gas, PR_gas | 04_fluid_sub.png |
| 4 | Synthetic Seismogram | Depth to TWT via interval Vp; Ricker / Ormsby wavelets; zero-offset RC & convolution; Shuey 2-term pre-stack gather 0-45 deg; AVO Intercept R0 & Gradient G sections; well-tie panel | 05-09 *.png per well |
| 5 | Multi-Well Overlays | AI vs Vp/Vs crossplot (brine + gas) for all wells on one figure; single-track log overlays (VP, GR, AI, Vp/Vs); colour-coded by well name | multi_well/ *.png |

# 6. Output Figures

Each well produces 9 figures plus a rock_physics.csv. The full output directory structure is shown below.

outputs/

├── F02-1/                       ← Real .wll data

│   ├── 01_log_overview.png      # Multi-track log QC

│   ├── 02_elastic_params.png    # AI, Vp/Vs, PR, K, G tracks

│   ├── 03_crossplots.png        # AI vs Vp/Vs and Poisson's ratio

│   ├── 04_fluid_sub.png         # Brine vs gas comparison

│   ├── 05_zero_offset.png       # Zero-offset wiggle synthetic

│   ├── 06_gather_brine.png      # Pre-stack gather (wiggle + VA)

│   ├── 06_gather_gas.png        # Pre-stack gather gas case

│   ├── 07_diff_avo.png          # Difference section + R0/G traces

│   ├── 08_wavelet_comparison.png # Ricker 25/40/60 Hz + Ormsby

│   ├── 09_well_tie.png          # GR→AI→RC→Brine→Gas panel

│   └── rock_physics.csv

├── F03-2/   (same structure)    ← Real .wll data

├── F03-4/   (same structure)    ← Real .wll data

├── F06-1/   (same structure)    ← Real .wll data

└── multi_well/

├── crossplot_brine_gas.png  # All wells, AI vs Vp/Vs

├── log_overlay_VP.png

├── log_overlay_GR.png

├── log_overlay_AI.png

└── log_overlay_VPVS.png

## CSV Column Reference

| Column | Units | Description |
| --- | --- | --- |
| DEPTH | m | Depth below KB on 0.15 m grid |
| VP / VS | m/s | P- and S-wave velocities (QC-cleaned) |
| RHOB | g/cc | Bulk density |
| GR | API | Gamma Ray |
| PHIT | fraction | Total porosity |
| AI / SI | 10³ kg/m²s | Acoustic and shear impedance |
| VPVS / PR | dimensionless | Vp/Vs ratio and Poisson's ratio |
| K_SAT / G_SAT | GPa | Saturated bulk and shear moduli |
| VP_gas / VS_gas | m/s | Gas-substituted P- and S-wave velocities |
| AI_gas / VPVS_gas | as above | Gas-substituted AI and Vp/Vs |
| RHOB_gas | g/cc | Gas-substituted density (mass-balance) |
| K_dry | GPa | Dry-frame bulk modulus (Gassmann back-strip) |

# 7. Adding a New Well

The pipeline is designed for zero-code extensibility. For a real well with .wll files:

- Copy all .wll files for the new well into the --wll_dir directory.

- Ensure filenames contain the well identifier (e.g. F04-1) — the auto-discovery regex F\d{2}-\d will match automatically.

- Add the well name to REAL_WELLS in run_pipeline.py:

# In run_pipeline.py

REAL_WELLS = ['F02-1', 'F03-2', 'F03-4', 'F06-1', 'F04-1']   # <- add here

- Run the pipeline with the new well name:

python run_pipeline.py --wells F02-1 F03-2 F03-4 F06-1 F04-1

# 8. F3 Block Geology & Well Summary

The F3 Block (Netherlands sector, North Sea) contains Pliocene-Pleistocene unconsolidated sand reservoirs overlain by Holocene marine clays. The gas sands exhibit Class III AVO (negative intercept, negative gradient) due to the low acoustic impedance contrast at the sand top.

| Well | Data Type | Depth Range | AI Change (Brine→Gas) | Vp/Vs Change |
| --- | --- | --- | --- | --- |
| F02-1 | Real .wll | 50–1500 m | −40% (4299 → 2596) | −35% (1.82 → 1.19) |
| F03-2 | Real .wll | 30–2140 m | −31% (5437 → 3759) | −28% (2.26 → 1.64) |
| F03-4 | Real .wll | 30–1900 m | −44% (4431 → 2474) | −40% (1.97 → 1.18) |
| F06-1 | Real .wll | 29–1700 m | −39% (4522 → 2764) | −35% (1.86 → 1.21) |

# 9. Rock Physics Theory

## Gassmann (1951) Fluid Substitution

The 3-step Gassmann workflow implemented in rock_physics.py:

### Step 1 — Back-strip original fluid to recover K_dry

K_dry = [K_sat·(φ·K_min/K_fl + 1 − φ) − K_min]

──────────────────────────────────────────

φ·K_min/K_fl + K_sat/K_min − 1 − φ

### Step 2 — Forward Gassmann with new fluid (K_fl_new)

K_sat_new = K_dry + (1 − K_dry/K_min)²

────────────────────────────────────────

φ/K_fl_new + (1−φ)/K_min − K_dry/K_min²

### Step 3 — Update density and recompute velocities

ρ_new  = ρ_dry + φ · ρ_fl_new

Vp_new = √( (K_sat_new + 4G/3) × 10⁹ / ρ_new )

Vs_new = √( G × 10⁹ / ρ_new )          ← fluid-independent

Assumptions: Low-frequency (seismic) limit · Homogeneous isotropic monomineralic frame · Fully connected equilibrated pore space · No chemical fluid-frame interaction.

## Shuey (1985) 2-Term AVO

R(θ) = R₀ + G · sin²(θ)

R₀ = ½ (ΔVp/V̄p + Δρ/ρ̄)                            [Intercept]

G  = ½ [ΔVp/V̄p − 4(V̄s/V̄p)²·(2ΔVs/V̄s + Δρ/ρ̄)]   [Gradient]

Valid for angles < 30°. The intercept-gradient crossplot places gas sands in Quadrant III (R₀ < 0, G < 0) — the classic Class III AVO anomaly characteristic of the F3 Block unconsolidated sands.

# 10. References

- Gassmann, F. (1951). Uber die Elastizitat poroser Medien. Vierteljahrsschrift der Naturforschenden Gesellschaft in Zurich, 96, 1–23.

- Shuey, R. T. (1985). A simplification of the Zoeppritz equations. Geophysics, 50(4), 609–614.

- Smith, T. M., Sondergeld, C. H., & Rai, C. S. (2003). Gassmann fluid substitutions: A tutorial. Geophysics, 68(2), 430–440.

- Mavko, G., Mukerji, T., & Dvorkin, J. (2009). The Rock Physics Handbook (2nd ed.). Cambridge University Press.

- dGB Earth Sciences. OpendTect F3 Demo Dataset. open.source.opendtect.org

F3 Block Rock Physics Framework  ·  F02-1 | F03-2 | F03-4 | F06-1 (All Real .wll Data)  ·  North Sea  ·  2026